HV Tail glueing

(please go through carefully and check the boxes )

- Place the sensor on the jig with the strip side facing down.

- The arrow on the sensor should be on the same side as the HV tail, thus pointing as indicated in picture 1.

- Push the sensor towards the stops as indicated in picture 1.

- Secure the sensor by switching on the vacuum.

- Place the HV tail:

  - The hole in the HV tail fits through the middle pin (pin2, see picture 2).

  - Push the HV tail leg towards pin1, secure it with the green pusher and fasten the screw.

- Apply glue:

  - Lift the HV tail with tweezers.

  - Apply a drop of glue on the tail below the bond pad.

  - When lowering the tail onto the sensor backplane make sure the tail is still in position.

- Slide the aluminum weight piece onto jig. The nose should push the HV tail down on the wire bond pad position as shown in picture 3.

- Let the glue cure.

- Remove the sensor with the HV tail from the jig by:

  - Lifting the weight piece,

  - un-fastening the screw of the pusher and moving the pusher aside,

  - carefully remove the three pins,

  - lift the sensor with a vacuum tool.

stops

Picture 1

pin1

pin2

pin3

Picture 2

Picture 3